1/1/08

Listed below are modifications of Asterisk in chan_sip.c and pbx.h to accommidate rudimentary implementation of Broadsoft Extensions.  The specifiactions in "BROADWORKS SIP ACCESS SIDE EXTENSIONS INTERFACE SPECIFICATIONS -RELEASE 13.0" were used, with the short term goal of achieving SLA functionality for Sipura phones (tested on SPA942's) .

Currently I have full SLA compatability with SPA942's working as well as integration with other (AASTRA 53i) SLA compatible phones.

To Asterisk:

In pbx.h:

AST_EXTENSION_REQLINESEIZE was added to ast_extension_states. A Broadsoft shared extension can request a line seize.

In chan_sip.c:

Added subscriptiontype of CALL_INFO.  The Call-Info header is specified in RFC 3261.

Added to the sip_user configuration and sip_pvt declarations "notify_exten" as discussed earlier in the fall on hte developers list. Originally this was going to be used due to lack of a subscription setup on the Sipura phones; Current firmware does generate a subscription to the peer exten automatically now.  I am using this variable to specifically direct the incoming call when a SLA is already active, otherwise the INVITE generated by the broadsoft generates an "incomplete address" from Asterisk.  Should this field be renamed to "sla_connect" ?

Transmit_state_notify subroutine was added and modified to handle CALL_INFO .  Also added state "LINE_SEIZE" used by Broadsoft and  AST_EXTENSION_REQLINESEIZE handler was added.  When a Broadsoft Shared extension requests a line seize, it is automatically granted. When other Broadsoft Shared extensions are sent a notification, there are told the the call is on "hold" (not private). This allows them to join into the SLA if their line key is hit.

Handle_request_invite subroutine was modified. When a Broadsoft Shared extension attempts to INVITE into an active shared call, the address was incomplete. This is checked for and the address is "completed" using the information from the "notify_exten" variable.

Handle_request_subscribe subroutine was modified. Deal with subscription for Line-Seize event and for Call-info subscriptions. Uses Call-info as the subscription type for Linksys user agents.